## Master Question for Notes

A main memory system has 8 modules, each 1 word wide.
The system bus can transfer 1 word in 100 ns (bus time).
Once a module receives a request, it remains busy for an additional 500 ns internally (internal time), during which it cannot accept a new request.

Only one request can be on the bus at a time.
The CPU can issue requests without delay, unless otherwise specified.

Total available time: 1 ms = 1,000,000 ns.

### Parts to Cover All Cases

- Initiations (Bus-limited case)
Find the maximum number of write initiations possible in 1 ms.

- Completions (Module-limited case)
Find the maximum number of completions possible in 1 ms.

- No Overlap
Assume internal module operations cannot overlap with others.
Find maximum write initiations in 1 ms.

- CPU-Limited
Same as (1), but CPU issues a request only once every 150 ns.
Find maximum initiations in 1 ms.

- Mixed Reads/Writes
Reads: bus 60 ns, internal 400 ns.
Writes: bus 100 ns, internal 500 ns.
70% of requests are reads. Find maximum initiations in 1 ms.

- Multi-word Transfers
Each request writes 4 words, bus transfer rate = 1 word per 40 ns.
Internal time per request = 500 ns.
Find maximum initiations in 1 ms.

- Percentage Utilization
In case (1), find bus utilization percentage at maximum initiations.

### Answers (for your notes)

Let:

- TbT_bTb​ = bus time/request

- TmT_mTm​ = module busy time/request (bus + internal)

- NNN = number of modules = 8

- TtotalT_{total}Ttotal​ = total time = 1,000,000 ns

(1) Initiations:
Bus-limited:

TtotalTb=1,000,000100=10,000\frac{T_{total}}{T_b} = \frac{1{,}000{,}000}{100} = 10{,}000Tb​Ttotal​​=1001,000,000​=10,000

Module capacity = 1,000,000600×8≈13,328\frac{1{,}000{,}000}{600} \times 8 \approx 13{,}3286001,000,000​×8≈13,328 → bigger than bus limit.
Answer: 10,000

(2) Completions:
Per module: 1,000,000/600≈1,6661{,}000{,}000 / 600 \approx 1{,}6661,000,000/600≈1,666.
Total: 1,666×8=13,3281{,}666 \times 8 = 13{,}3281,666×8=13,328 completions possible if 13,328 initiations could happen — but bus only starts 10,000 in 1 ms.
So completions = 10,000 (bus still bottleneck here).

(3) No Overlap:
Time per request = 100 + 500 = 600 ns per system (not per module).
Max = 1,000,000/600≈1,6661{,}000{,}000 / 600 \approx 1{,}6661,000,000/600≈1,666.
Answer: 1,666

(4) CPU-Limited:
CPU issues every 150 ns.
Max initiations = 1,000,000/150≈6,6661{,}000{,}000 / 150 \approx 6{,}6661,000,000/150≈6,666.
Bus and modules can handle more → CPU is bottleneck.
Answer: 6,666

(5) Mixed Reads/Writes:
Avg bus time = 0.7×60+0.3×100=720.7 \times 60 + 0.3 \times 100 = 720.7×60+0.3×100=72 ns.
Bus-limited max = 1,000,000/72≈13,8881{,}000{,}000 / 72 \approx 13{,}8881,000,000/72≈13,888.
Check module limits (both reads and writes fit easily) → bus bottleneck.
Answer: 13,888

(6) Multi-word Transfers:
Bus time = 4×40=1604 \times 40 = 1604×40=160 ns.
Bus-limited max = 1,000,000/160=6,2501{,}000{,}000 / 160 = 6{,}2501,000,000/160=6,250.
Module capacity: 1,000,000/660×8≈12,1211{,}000{,}000 / 660 \times 8 \approx 12{,}1211,000,000/660×8≈12,121 → bus bottleneck.
Answer: 6,250

(7) Percentage Utilization:
From (1): Total bus time = 10,000×100=1,000,00010{,}000 \times 100 = 1{,}000{,}00010,000×100=1,000,000 ns.
Utilization = 1,000,000/1,000,000×100%=100%1{,}000{,}000 / 1{,}000{,}000 \times 100\% = 100\%1,000,000/1,000,000×100%=100%.
Answer: 100%